| ## Workplace Address |  |  |  |  |  |  |  |  | Date |  | / / |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INSPECTION TEAM NAMES |  |  |  |  |  |  |  |  |  |  |  |  |
| HARNESS Type & Serial No |  |  | Manufacturer |  |  |  |  |  |  |  |  |  |
|  |  |  | Serial No |  |  |  |  |  |  |  |  |  |
| All safety harnesses including associated lanyards must be inspected before use and every 6 months by a competent person. Harness includes all attachments e.g. lanyards, retracting lanyards, shock absorbers, etc. After a fall, the subject safety harness and lanyards to be immediately withdrawn from use. |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect items | Look at these item details and any other details that may cause hazards |  |  |  | Is item OK, Not OK or N/A |  |  | Action to fix items & by whom |  |  |  | Fix by Date |
| Certification | Current & to Australian Standards |  |  |  | OK | Not | N/A |  |  |  |  |  |
| & data tag |  |  |  |  |  |  |  |  |  |  |  |  |
| Care | All items clean, no visual wear & tear |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | All items properly stored and dry |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | All items maintained |  |  |  | OK | Not | N/A |  |  |  |  |  |
| Webbing | No cut or tears |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No abrasion damage |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No excessive stretching |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No damage - heat, corrosives solvents |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No deterioration - rotting, mildew, UV |  |  |  | OK | Not | N/A |  |  |  |  |  |
| Hardware – | No distortion of hook or latch |  |  |  | OK | Not | N/A |  |  |  |  |  |
| snap hooks & | No cracks or forging folds |  |  |  | OK | Not | N/A |  |  |  |  |  |
| karabiners | No wear at swivels & latch points |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No open rollers |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | Free latch movement over full travel |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No broken, weak, misplaced latch springs |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No dirt, rust or foreign obstruction |  |  |  | OK | Not | N/A |  |  |  |  |  |
| D Ring | No excessive vertical movement of the straight portion of the D-ring at its attaching point |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No cracks at intersection of straight & curved portions |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No distortion or other physical damage |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No excessive loss of cross section |  |  |  | OK | Not | N/A |  |  |  |  |  |
| Buckles & | No distortion or physical damage |  |  |  | OK | Not | N/A |  |  |  |  |  |
| adjusters | No cracks & forging laps |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No bent tongues |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  | No open rollers |  |  |  | OK | Not | N/A |  |  |  |  |  |
|  |  |  |  |  | OK | Not | N/A |  |  |  |  |  |
| OTHER ITEMS | Sub Item |  |  |  |  |  |  | Comments |  |  |  | Date |
| Issues raised & Action taken |  |  |  |  |  |  |  | Copies to |  |  |  |  |
|  |  |  |  |  |  |  |  | Up-line Manager |  |  |  |  |
|  |  |  |  |  |  |  |  | HSR |  |  |  |  |
|  |  |  |  |  |  |  |  | OHS Committee |  |  |  |  |
| Operator / Manager |  |  |  |  |  |  |  | Date |  | / / |  |  |