| ## Workplace Address |  |  |  |  |  |  |  |  |  |  | Date |  |  | / / |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INSPECTION TEAM NAMES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STEP / LADDER |  |  |  | Material | Step Platform |  |  |  | Step |  |  |  | Ladder |  |  |
| Type & Serial No |  |  |  | Type (circle) | Metal |  |  |  | Fibreglass |  |  |  | Timber |  |  |
|  |  |  |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Serial No |  |  |  |  |  |  |  |  |  |  |  |
| Portable steps & ladders to comply with Australian Standard 1892:2018 for metal timber or reinforced plastic ladders. Portable steps & ladders to be correctly selected and used for the task to be undertaken. Refer FORM Ladder Risk Assessment to assist assessment of risk, selection and use of portable ladder. Defective portable steps & ladders must be immediately removed from use. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect items | Look at these item details and any other details that may cause hazards |  |  |  |  | Is item OK, Not OK or N/A |  |  |  | Action to fix items & by whom |  |  |  |  | Fix by Date |
| General | Industrial grade, suitable weight rating |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | Has not been modified |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No loose steps or rungs |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No loose screws, bolts, fixtures, etc |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No cracked split or broken items |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | Rungs, cleats, steps uniformly spaced |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No rungs, steps, rails missing |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | Free of grease, oil, slippery materials |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No splinters on timber parts |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No coatings on timber ladders |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No loose joints on rungs, side rails, etc |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | All moveable parts operate freely |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No damaged, worn nonslip feet |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No cracks, splits to rails |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No loose nails, screws |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No other items attached |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
| Step & Step | Opens & sits square & firm |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
| Platforms | Does not exceed AS 1892 dimensions |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No loose, bent hinge spreaders |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No sharp edges or spreaders |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | No loose hinges |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
| Extension | No loose, broken or missing locks |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
| Ladders | No defective locks |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | Locks fully seat when extended |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  | Does not exceed AS 1892 dimensions |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
|  |  |  |  |  |  | OK | Not | N/A |  |  |  |  |  |  |  |
| OTHER ITEMS | Sub Item |  |  |  |  |  |  |  |  | Comments |  |  |  |  | Date |
| Issues raised & Action taken |  |  |  |  |  |  |  |  |  | Copies to |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Up-line Manager |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | HSR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | OHS Committee |  |  |  |  |  |
| Operator / Manager |  |  |  |  |  |  |  |  |  | Date |  | / / |  |  |  |